# Literature Review:

# 1. Introduction

The demand for optical networks has been rising for many years, the growth in internet traffic has fundamentally transformed network requirements. Applications such as real-time video streaming, cloud computing, AI workloads, and IoT communication and their demand of bandwidth has been increasing over time because modern digital life generates massive amounts of data. Traditional electronic networks are not sufficient for this increasing demand of bandwidth.

1.1 Optical switching:

Optical switching is the process of transmitting optical signals using light without converting them into electrical signals. As optical fibers offer terabits per second throughput using Wavelength Division Multiplexing (WDM), researchers have explored various optical switching paradigms including Optical Circuit Switching (OCS), Optical Packet Switching (OPS), and Optical Burst Switching (OBS).

1.2 Optical Circuit switching (OCS):

In this optical switching the transmission is via dedicated light path between the source and the destination allowing information to travel end to end without being converted into electrical signals along the way. Once the path is reserved, no one can use it until the connection ends. This approach provides very high bandwidth and low delay because the optical signal stays in the light domain throughout the route. Furthermore, since no buffering takes place, the capacity of the circuit must equal the peak data rate.

The issue in this technology is poor resource utilization, because dedicated lightpaths must be reserved for the entire duration of a communication session even when the channel is idle, bandwidth remains unused. OCS also has long setup times, since establishing an end-to-end optical circuit requires signaling and configuration across all intermediate nodes.

1.3 Optical Packet switching (OPS):

Optical packet switching is a method of sending optical signals through optical network by dividing information into packets and routing each one independently using optical components. Instead of setting up a dedicated light path, the network forwards packets based on their headers like electronic routers.

The incoming optical signal is first converted into an electrical signal because current network devices cannot directly process optical data; there is no optical RAM or mature optical processing technology. The packet header is processed electronically to make routing decisions, and then the signal is converted back into optical form for high-speed transmission across the fiber. Although OPS offers fine granularity, fast switching, and high bandwidth utilization, it suffers from contention due to limited buffering and frequent optical-electrical-optical (OEO) conversions. These limitations make OPS less efficient and less mature compared to more advanced optical techniques like OBS.

1.4 Optical Burst Switching (OBS):

In OBS, a control packet is sent first to set up a connection (by reserving an appropriate wavelength and configuring the switches along a path), followed by a burst (e.g., more than one data IP packets) of data without waiting for an acknowledgement for the connection establishment. In other words, OBS uses one way reservation protocols like tell-and-go (TAG) [E. Varvarigos and V. Sharma. The ready-to-go virtual circuit protocol: A loss-free protocol for multigigabit networks using FIFO buffers. IEEE/ACM Transactions on Networking, 5(5):705{ 718, October 1997.], also known in ATM as fast reservation protocol (FRP) or ATM Block Transfer with Immediate Transmissions (or ABT-IT) [H. Shimonishi, T. Takine, M. Murata, and H. Miyahara. Performance analysis of fast reservation protocol with generalized bandwidth reservation method. In Proceedings of IEEE Infocom, volume 2, pages 758{767, 1996.] reservation protocols.

OBS also differs from optical packet switching mainly in that the former can switch a burst of several packets to a (short) session using one control packet, thus resulting in a lower control overhead per data unit.

OBS uses out-of-band signaling, and unlike packet switching. They can be separated by an offset time at the source and at intermediate nodes, meaning the control packet is sent ahead to set the path, while the data burst follows it. By choosing the offset time at the source to be larger than the total processing time of the control packet along the path [M. Yoo and C. Qiao. Just-enough-time (JET): a high-speed protocol for bursty trac in optical networks. In IEEE/LEOS Technologies for a Global Information Infrastructure, August 1997.] one can eliminate the need for a data burst to be buffered at any subsequent intermediate node just to wait for the control packet to get processed.

In OBS, the wavelength on a link used by the burst will be released as soon as the burst passes through the link, either automatically according to the reservation made (as in JET) or by an explicit release packet. In this way, bursts from different sources to different destinations can effectively utilize the bandwidth of the same wavelength on a link in a time-shared, statistically multiplexed fashion. Note that, in case the control packet fails to reserve the bandwidth at an intermediate node, the burst (which is considered blocked at this time) may have to be dropped.

The OBS is balance between switching coarse-grained optical circuits and switching fine-grained optical packets and combines the best of both paradigms, as illustrated in Table 1.

A Comparative Study of OCS, OPS, and OBS in Modern Optical Networks:

OCS sets up dedicated end-to-end lightpaths before data transmission, giving high reliability and guaranteed bandwidth but wasting the bandwidth when the path is idle, OPS transfer each packet individually and utilizing the bandwidth efficiently because it uses statistical multiplexing (The wavelength is shared dynamically among packets coming from different users) instead of reserving a fixed channel, but requiring extremely fast optical processing, which is still technologically challenging, OBS sits between OCS and OPS by grouping packets into bursts and sending a control packet ahead to set up the path. This gives it better efficiency than OCS and lowers hardware complexity than OPS. Its burst-based approach provides coarse granularity compared to OPS but still offers finer granularity than OCS, making OBS a practical middle-ground for high-speed optical networks.

| Optical Switching | Bandwidth Utilization | Latency | Optical Buffer | Proc./Sync. Overhead | Adaptivity |
| --- | --- | --- | --- | --- | --- |
| Circuit | Low | lower | Not required | Low | Low |
| Packet | High | Low | Required | High | High |
| Burst | Higher | Lower | Not required | Lower | Higher |

Table 1: A comparison between three optical switching paradigms

2. OBS Network Architecture and Functioning:

The architecture of Optical Burst Switching is based on a buffer-less WDM network that provides incredibly huge bandwidth for communication. An OBS network is composed of three primary types of nodes: ingress nodes, egress nodes, and core nodes (or routers).

2.1. Ingress Node Functionality (Edge Layer)

The ingress node is located at the edge of the optical core and acts as the interface between the electronic client network and the all-optical OBS core. The ingress node is the only part of the network that holds buffering capability in the buffer-less environment to speed up data transfer. A crucial function is the burst assembly process, where packets arriving are aggregated into a single, larger data unit called a Data Burst (DB). Once a Data Burst is assembled, the ingress node generates a Burst Control Packet (BCP). The BCP contains all the necessary routing and control information for its corresponding DB. After the BCP is transmitted, the Data Burst follows on a wavelength after a predetermined time interval known as the offset time. This offset time allows core nodes sufficient time to process the BCP and configure the optical switch before the DB arrives.

2.2. Core Node Functionality (Optical Core Layer)

The optical core layer consists of several interconnected OBS core routers or switches. Core nodes are designed to be buffer-less and perform all-optical switching. The core node receives the BCP, process it electronically to determine the required output port and same wavelength, and then configures the optical switch. The key functional benefit of OBS is that the Data Burst (DB) itself is switched entirely into the optical domain, avoiding the need for optical-to-electronic conversion at intermediate nodes.

2.3. Egress Node Functionality

The egress node is responsible for receiving the Data Burst from the optical core and converting the optical signal back into individual packets for delivery to the client network. This involves performing the necessary optical-to-electronic conversion.

Figure 1:Architecture-of-optical-burst-switching-network

3. Burst Reservation Protocols in Optical Burst Switching (OBS)

Burst reservation protocols dictate how wavelengths are reserved for a data burst and subsequently released. Optical Burst Switching (OBS) has adopted its burst reservation scheme from the well-known ATM Block Transfer (ABT) scheme [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. Like ABT, OBS employs two main versions: reservation with delayed transmission and reservation with immediate transmission [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

3.1 Delayed Reservation Schemes

In the delayed reservation scheme, the control packet (BCP) and the payload (Data Burst) are separated in offset time [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. This offset time is crucial as it accommodates the processing of the control packet at the intermediate core nodes [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. An output wavelength is reserved for the data burst just before the arrival of the first bit of the data burst means the wavelength is reserved exactly when the data burst arrives which effectively eliminates the requirement of optical buffering at intermediate nodes in the core [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. This lack of optical buffering simplifies the OBS network architecture [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. The burst blocking probability in such networks can be reduced by employing partial wavelength conversion at the core nodes. However, if no wavelength can be reserved at the appropriate time upon arrival of the setup message, the setup message is rejected, and the corresponding data burst is dropped [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

3.2 Immediate Reservation Schemes

The immediate reservation scheme reserves an output wavelength for a payload immediately after the arrival of the corresponding control packet [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. Like the delayed scheme, if a wavelength cannot be reserved at that time, the setup message is rejected, and the corresponding data burst is dropped [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

4. Specific Burst Reservation Protocols (TAG, TAW, JIT, JET)

Different specific burst reservation protocols have been proposed in the literature, including Tell-and-Go (TAG), Tell-and-Wait (TAW), Just-in-Time (JIT), and Just-enough-Time (JET) [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

4.1 Tell-and-Go (TAG): In the TAG approach, the sender transmits the control packet and does not wait for any Acknowledgement (ACK) regarding resource reservation [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

4.2 Tell-and-Wait (TAW): With TAW, the ingress node attempts to reserve the wavelength from the source to the destination [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. If the reservation on all links is successful, an ACK is received; otherwise, a NAK (Negative Acknowledgement) is received [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. Upon receiving an ACK, the burst is transmitted, and in the case of a NAK, the node initiates a retransmission after a backoff time [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

4.3 Just-in-Time (JIT): The initially proposed JIT requires the ingress node to request a central scheduler for each burst transmission, and this scheduler reserves the resources in advance for the burst [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. This centralized control makes the approach not scalable [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

4.4 Just-enough-Time (JET): JET is one of the most important distributed protocols for OBS networks that does not require any buffering of the payload or delay at intermediate nodes [Contention avoidance scheme using machine learning inspired deflection routing.pdf]. This protocol utilizes the offset time, and resources are reserved from the arrival time of the burst instead of the arrival time of the control packet [Contention avoidance scheme using machine learning inspired deflection routing.pdf].

5. Burst Assembly Strategies and Their Performance Effects

The burst assembly mechanism at the ingress node significantly impacts the overall network performance, as it dictates the size and timing of the Data Bursts entering the core. Burst assembly is a challenging process for efficient utilization in OBS networks.

Two primary parameters govern burst aggregation: the maximum waiting time (timer value) and the minimum size (threshold value) of the burst. Based on these, assembly algorithms are broadly classified as timer-based, threshold-based, or hybrid.

5.1 Timer-Based Assembly: In this scheme, a timer is initiated at the start of the first packet or immediately after a preceding burst is sent. The burst is generated when this timer expires, containing all accumulated packets in the buffer at that instant.

5.2 Threshold-Based Assembly: A burst is triggered and sent when the count of aggregated packets in the buffer reaches a pre-defined threshold value.

5.3 Hybrid Assembly: These approaches combine the characteristics of both timer-based and threshold-based assembly to balance delay and burst size.

7. Contention Resolution Techniques

The buffer-less nature of OBS core switches makes the network highly susceptible to data contention issues. Contention occurs when two or more Data Bursts are competing for the same output wavelength at the same time. This leads to the undesirable event of burst loss, which severely degrades overall network performance. Effectively resolving contention is the main challenge in achieving a low burst loss probability (BLP).

Fiber delay lines (FDL) is one of the  solution for contention resolution in OBS, used when bursts are delayed in core nodes due to the lack of random-access memory. However, using FDL results in shorter delays.

Another possible contention resolution strategy is deflection routing, where the optical burst is sent to the next output wavelength of a predetermined core node.

8. Burst Segmentation and Dropping

When a contention event occurs, instead of dropping the entire Data Burst, only the conflicting portion of the burst (the segments that overlap with the occupied channel) is dropped, allowing the non-conflicting segments to continue transmission.

Burst segmentation is recognized as a specific approach for effectively reducing packet loss in OBS networks.

Figure 2:Selective Segment dropping for two contending bursts

9. Quality of Service (QoS) Provisioning

Provisioning Quality of Service (QoS) in Optical Burst Switching networks is a necessity for supporting differentiated services. QoS involves differentiating traffic to ensure high-priority traffic experiences lower loss or delay [A. Kumar Garg, “Heuristic solutions related to challenges of optical burst switching paradigm”, Journal of Microwaves, Optoelectronics and Electromagnetic Applications, vol. 10, no. 2, 2011.].

Performance Evaluation and Simulation Modeling

Accurate performance analysis of OBS network is vital for the development and future deployment. OBS is still in the research and experimental stage, modeling techniques and simulation tools remain critical for evaluating newly proposed architectural designs.

The OMNeT++ For the OBS:

In previous OBS research, OMNeT++ has been extensively used as the primary simulation environment to study and address OBS-related challenges, including burst assembly delays, signaling overhead, and contention resolution. The well-known OBS models developed by the Public University of Navarre and the University of Algarve were implemented on earlier OMNeT++ releases and extensively used to evaluate routing and scheduling mechanisms in optical networks. [ “An OMNeT++ Model for the Evaluation of OBS Routing Strategies”] . However, these simulation frameworks were designed for legacy versions of OMNeT++ that rely on outdated libraries, deprecated APIs, and GUI components no longer supported by modern operating systems. As a result, reproducing, extending, or validating the earlier OBS experiments has become increasingly difficult, creating a clear gap in current research. This motivates the need for a modernized, fully compatible simulation test bed using the latest stable release, OMNeT++ 6.2, which introduces a redesigned event scheduler, updated NED syntax, and improved modularity. Developing an updated OBS simulation environment in OMNeT++ 6.2 is therefore essential not only to revive and extend prior work but also to enable future researchers to explore OBS architectures, signaling constraints, and routing strategies on a stable and maintainable platform. OMNeT++ has been successfully adopted as a platform for developing, implementing, and assessing these models.1

Definition and Core Features of the OMNeT++ Framework

OMNeT++ is an advanced discrete event simulation system that provides a component-based, modular, and open-architecture framework. Primarily implemented in C++, it is a simulation library designed for building network simulators.

The core strength of the OMNeT++ environment lies in its modular structure, which facilitates the easy integration of new functional modules and the modification of existing components.

2.3.2 Design Implementation: C++, NED Language, and INI

The construction of network models within OMNeT++ leverages (use ) both traditional programming(C++) and declarative(NED file) specification methods.

- Source File (C++ Implementation): Complex operational mechanisms and network techniques, such particularly the scheduling logic and node processing functions inherent in OBS, are defined and implemented using C++. OMNeT++ is fundamentally a C++ library and framework.

- Network Description (NED) Language: The overall architecture and topological arrangement of the network how modules like EdgeNode and CoreNode are interconnected are specified using the NED language. This descriptive language facilitates the definition of compound modules and hierarchical network structures.

- INITIALIZATION FILE(.ini file):

The omnetpp.ini file in OMNeT++ is used to set up and control the simulation. Unlike NED files that define the network structure and C++ files that describe module behavior, the INI file lets you configure parameters like packet size, buffer size, link delays, and simulation time limits. It also allows you to choose which network to run and manage outputs such as log or result files. You can even create multiple sections in the file to test different scenarios without changing the NED or C++ code, which makes running experiments and comparing results much easier.

2 The Role of Model Framework:

The component-based and modular nature of OMNeT++ is simultaneously its greatest asset. Prior successful OBS models relied heavily on custom components (C++ modules) integrated with specific versions of the OMNeT++  . Upgrading the simulation platform to a modern release, such as OMNeT++ 6.2, requires extensive refactoring of this custom OBS code to accommodate potential changes and incompatibilities in the Application Programming Interfaces (APIs) of the new core libraries. This architectural interdependence necessitates the proposed development effort to create a stable, updated simulation environment.

Implementation and Evaluation of Prior OBS Models using OMNeT++

Significant prior research has used OMNeT++ to design and analyse OBS networks, mainly to study how different network configurations influence key performance metrics such as burst loss.

Modeling Node Architecture and Internal Components

Previous OMNeT++ OBS models defined the network structure using two primary compound modules: the EdgeNode and the CoreNode, faithfully replicating the theoretical OBS architecture.

A. Edge Node Internal Structure

The ingress EdgeNode utilizes specialized submodules to manage traffic aggregation, path selection, and signaling preparation

- Burst Gen: This module generates the traffic, modeling bursts as OMNeT++ messages based on a Poisson process and threshold-based assembly.

- Routing Table: Stores the optimized path information. Importantly, in specialized studies, this table was loaded with complete path information derived from an external optimization process  during the simulation's initialization phase.

- Dispatcher: Manages burst queues organized by destination address and calculates the necessary offset time based on the estimated processing delay per core node and the total number of hops the CPH must traverse.

- EdgeChSched: Functions as the output port driver, responsible for transmitting the CPH and the subsequent data burst onto the core backbone after ensuring a free wavelength has been selected.

The egress EdgeNode utilizes specialized submodules to manage burst reception, data reassembly, and packet-level delivery.

- BurstReceiver: Receives incoming bursts and separates data from control information.

- ReassemblyBuffer: Stores burst pieces temporarily and puts them back in order.

- BurstDecoder / Segmentation Handler: Converts bursts back into normal packets.

- TrafficSink: Receives final packets and records performance statistics.

B. Core Node Internal Structure

The CoreNode is designed to handle control packet processing, switching, and contention resolution :

- InCorePortDrv: The input port driver that receives traffic and forwards CPHs and bursts to the SwitchUnit.

- SwitchUnit: This module is critical, executing the OEO conversion for CPHs (allowing processing) and the all-optical switching for data bursts. It models the assumed CPH processing delay and relies on an internal switching table.1

- CoreChSched: Acts as the output port driver for the core node. This is the designated location where resource contention takes place, as it checks for wavelength availability using flag systems. The module aggregates drop statistics, documenting burst loss due to failure in finding an available wavelength.