# Literature Review:

# 1. Introduction

The demand for optical networks has been rising for many years, the growth in internet traffic has fundamentally transformed network requirements. Applications such as real-time video streaming, cloud computing, AI workloads, and IoT communication and their demand of bandwidth has been increasing over time because modern digital life generates massive amounts of data. Traditional electronic networks are not sufficient for this increasing demand of bandwidth.

1.1 Optical switching:

Optical switching is the process of transmitting optical signals using light without converting them into electrical signals. As optical fibers offer terabits per second throughput using Wavelength Division Multiplexing (WDM), researchers have explored various optical switching paradigms including Optical Circuit Switching (OCS), Optical Packet Switching (OPS), and Optical Burst Switching (OBS).

1.2 Optical Circuit switching (OCS):

In this optical switching the transmission is via dedicated light path between the source and the destination allowing information to travel end to end without being converted into electrical signals along the way. Once the path is reserved, no one can use it until the connection ends. This approach provides very high bandwidth and low delay because the optical signal stays in the light domain throughout the route. Furthermore, since no buffering takes place, the capacity of the circuit must equal the peak data rate.

The issue in this technology is poor resource utilization, because dedicated lightpaths must be reserved for the entire duration of a communication session even when the channel is idle, bandwidth remains unused. OCS also has long setup times, since establishing an end-to-end optical circuit requires signaling and configuration across all intermediate nodes.

1.3 Optical Packet switching (OPS):

Optical packet switching is a method of sending optical signals through optical network by dividing information into packets and routing each one independently using optical components. Instead of setting up a dedicated light path, the network forwards packets based on their headers like electronic routers.

The incoming optical signal is first converted into an electrical signal because current network devices cannot directly process optical data; there is no optical RAM or mature optical processing technology. The packet header is processed electronically to make routing decisions, and then the signal is converted back into optical form for high-speed transmission across the fiber. Although OPS offers fine granularity, fast switching, and high bandwidth utilization, it suffers from contention due to limited buffering and frequent optical-electrical-optical (OEO) conversions. These limitations make OPS less efficient and less mature compared to more advanced optical techniques like OBS.

1.4 Optical Burst Switching (OBS):

In OBS, a control packet is sent first to set up a connection (by reserving an appropriate wavelength and configuring the switches along a path), followed by a burst (e.g., more than one data IP packets) of data without waiting for an acknowledgement for the connection establishment. In other words, OBS uses one way reservation protocols like tell-and-go (TAG) [E. Varvarigos and V. Sharma. The ready-to-go virtual circuit protocol: A loss-free protocol for multigigabit networks using FIFO buffers. IEEE/ACM Transactions on Networking, 5(5):705{ 718, October 1997.], also known in ATM as fast reservation protocol (FRP) or ATM Block Transfer with Immediate Transmissions (or ABT-IT) [H. Shimonishi, T. Takine, M. Murata, and H. Miyahara. Performance analysis of fast reservation protocol with generalized bandwidth reservation method. In Proceedings of IEEE Infocom, volume 2, pages 758{767, 1996.] reservation protocols.

OBS also differs from optical packet switching mainly in that the former can switch a burst of several packets to a (short) session using one control packet, thus resulting in a lower control overhead per data unit.

OBS uses out-of-band signaling, and unlike packet switching. They can be separated by an offset time at the source and at intermediate nodes, meaning the control packet is sent ahead to set the path, while the data burst follows it. By choosing the offset time at the source to be larger than the total processing time of the control packet along the path [M. Yoo and C. Qiao. Just-enough-time (JET): a high-speed protocol for bursty trac in optical networks. In IEEE/LEOS Technologies for a Global Information Infrastructure, August 1997.] one can eliminate the need for a data burst to be buffered at any subsequent intermediate node just to wait for the control packet to get processed.

In OBS, the wavelength on a link used by the burst will be released as soon as the burst passes through the link, either automatically according to the reservation made (as in JET) or by an explicit release packet. In this way, bursts from different sources to different destinations can effectively utilize the bandwidth of the same wavelength on a link in a time-shared, statistically multiplexed fashion. Note that, in case the control packet fails to reserve the bandwidth at an intermediate node, the burst (which is considered blocked at this time) may have to be dropped.

The OBS is balance between switching coarse-grained optical circuits and switching fine-grained optical packets and combines the best of both paradigms, as illustrated in Table 1.

A Comparative Study of OCS, OPS, and OBS in Modern Optical Networks:

OCS sets up dedicated end-to-end lightpaths before data transmission, giving high reliability and guaranteed bandwidth but wasting the bandwidth when the path is idle, OPS transfer each packet individually and utilizing the bandwidth efficiently because it uses statistical multiplexing (The wavelength is shared dynamically among packets coming from different users) instead of reserving a fixed channel, but requiring extremely fast optical processing, which is still technologically challenging, OBS sits between OCS and OPS by grouping packets into bursts and sending a control packet ahead to set up the path. This gives it better efficiency than OCS and lowers hardware complexity than OPS. Its burst-based approach provides coarse granularity compared to OPS but still offers finer granularity than OCS, making OBS a practical middle-ground for high-speed optical networks.

| Optical Switching | Bandwidth Utilization | Latency | Optical Buffer | Proc./Sync. Overhead | Adaptivity |
| --- | --- | --- | --- | --- | --- |
| Circuit | Low | lower | Not required | Low | Low |
| Packet | High | Low | Required | High | High |
| Burst | Higher | Lower | Not required | Lower | Higher |

Table 1: A comparison between three optical switching paradigms

2. Literature Review:

Previous research on Optical Burst Switching (OBS) includes several works that built simulation test beds using OMNeT++ 4.x. One major study is “OBS Network Model for OMNeT++: A Performance Evaluation” by Espina et al. (2010), which created a full OBS model supporting burst assembly, control channel processing, and WDM backbone behavior, giving researchers a flexible test bed for performance testing. Another important contribution is “An OMNeT++ Model for the Evaluation of OBS Routing Strategies” by Barradas and Medeiros (2008/2010), where the authors implemented an OMNeT++-based OBS simulator to test different routing strategies and showed how alternative path-selection can reduce burst loss rates. Later, the OBS Modules for OMNeT++ (2012–2013) extended this work by offering ready-to-use components for building OBS networks in OMNeT++ 4.x, allowing researchers to create test beds without implementing everything from scratch. Earlier tools like nOBS on NS-2 existed, but surveys such as the MECS-Press review (2013) reported that OMNeT++ provides better modularity and flexibility for OBS research. These works collectively form the foundation for using OMNeT++ as a simulation test bed for evaluating routing, contention resolution, network load behavior, and QoS in OBS networks.

In previous OBS research, OMNeT++ has been extensively used as the primary simulation environment to study and address OBS-related challenges, including burst assembly delays, signaling overhead, and contention resolution. The well-known OBS models developed by the Public University of Navarre and the University of Algarve were implemented on earlier OMNeT++ releases and extensively used to evaluate routing and scheduling mechanisms in optical networks. [ “An OMNeT++ Model for the Evaluation of OBS Routing Strategies”] . However, these simulation frameworks were designed for legacy versions of OMNeT++ that rely on outdated libraries, deprecated APIs, and GUI components no longer supported by modern operating systems. As a result, reproducing, extending, or validating the earlier OBS experiments has become increasingly difficult, creating a clear gap in current research. This motivates the need for a modernized, fully compatible simulation test bed using the latest stable release, OMNeT++ 6.2, which introduces a redesigned event scheduler, updated NED syntax, and improved modularity. Developing an updated OBS simulation environment in OMNeT++ 6.2 is therefore essential not only to revive and extend prior work but also to enable future researchers to explore OBS architectures, signaling constraints, and routing strategies on a stable and maintainable platform. OMNeT++ has been successfully adopted as a platform for developing, implementing, and assessing these models.

3. Problem Statement:

Optical Burst Switching (OBS) is still in the research domain, and currently, there is no experimental test bed available. To work on this technology, a test bed is essential, which can be provided in the form of a software-based simulation framework. Previous research was carried out by the Public University of Navarre, Spain, and the University of Algarve, Portugal, where an OBS network model for OMNeT++ was developed and evaluated. However, that work was implemented on older versions of OMNeT++, which are now obsolete and incompatible with modern operating systems, thus further research/evaluation seems difficult. Therefore, our work focuses on using the latest stable release, OMNeT++ 6.2, to develop an updated simulation environment for OBS.

4. Research Aims and Objectives

- The primary aim of this research is to design and implement a modern Optical Burst Switching simulation framework using OMNeT++ 6.2 that resolves compatibility issues and supports future optical network research.

- To achieve this aim, the study pursues the following specific objectives:

- To investigate the limitations of traditional optical switching methods such as OCS and OPS.

- To design a modular OBS network architecture including edge nodes, core nodes, burst assembly, signaling.

- To implement the OBS framework in OMNeT++ 6.2.

- To analyze and evaluate the performance of the developed simulation model through experimental scenarios.

5. Research Question:

How can Optical Burst Switching (OBS) and different burst-assembly mechanisms improve network performance under different traffic conditions?

Over the past few decades, the number of internet users has increased significantly, generating heavy traffic and a growing demand for bandwidth. Electronic core routers cannot handle this load efficiently, making optical networking a practical long-term solution. Existing Optical Circuit Switching (OCS) wastes bandwidth, while Optical Packet Switching (OPS) requires complex optical processing and specialized hardware. This makes Optical Burst Switching (OBS) a promising alternative. Since real optical hardware is not easily accessible, simulation provides an immediate and practical solution for research. Our study focuses on simulating an OBS network, particularly at the edge node where burst assembly occurs. The goal is to analyze different burst-assembly mechanisms and compare their effectiveness under various network conditions.

6. Limitations and Scope:

As an undergraduate student, attempting to implement a complete OBS network including core switching, contention resolution, and advanced routing strategies is beyond the scope of this study. Developing and testing all these components requires extensive expertise, and significant time for design and analysis. Our focus is on edge node specifically the burst-assembly process, which is a critical part of OBS performance. By concentrating on this manageable yet important component, we can gain meaningful insights into burst-assembly mechanisms and their impact on overall network efficiency.

6. Thesis Structure (Chapter Outline)

This thesis is divided into six chapters. Each chapter contributes to developing, implementing, and analyzing a modern Optical Burst Switching (OBS) simulation farmwork using OMNeT++ 6.2.

Chapter-by-Chapter Summary

Chapter 1 – Introduction

This chapter discusses the rapid growth in internet traffic and the limitations of traditional electronic networks. It introduces optical switching and explains why Optical Burst Switching (OBS) is a promising solution compared to OCS and OPS. The chapter defines the problem resulting from outdated OBS simulation models, presents the research aim, objectives, research question, and clearly states the study’s scope focusing on burst-assembly at the OBS edge node using OMNeT++ 6.2.

Chapter 2 – Literature Review

This chapter reviews fundamental concepts of optical switching, including OCS, OPS, and OBS. It compares these paradigms in terms of bandwidth utilization, latency, processing overhead, buffering, and optical-electrical conversion requirements. Previous OBS simulation studies are summarized especially the models built on older OMNeT++ versions (4.x). The chapter highlights incompatibility with modern operating systems. It identifies the gap in existing research: the absence of an updated OBS simulation framework compatible with OMNeT++ 6.2.

Chapter 3 – OMNET++

This chapter explains the design and development process of the OBS simulation environment. It describes the modular OBS architecture edge node, burst assembly, control packet handling, and basic WDM behavior. It details how OMNeT++ 6.2 is used to model the network, define NED components, configure simulation parameters, and set up burst-assembly mechanisms. The methodology includes the experimental setup used for evaluating the performance of these mechanisms.

Chapter 4 – Results and Performance Evaluation

This chapter presents the outcomes of the simulation experiments conducted on the developed OBS model. It evaluates network behavior under different traffic loads and analyzes burst-assembly performance. Key metrics such as burst formation delay, resource utilization, and burst handling efficiency are examined. The results show how different burst-assembly strategies impact overall OBS performance.

Chapter 5 – Discussion

This chapter interprets the results and relates them to existing literature. It explains how the findings align with or differ from earlier OBS studies. The chapter discusses the effectiveness of various burst-assembly mechanisms and the practical relevance of the developed OMNeT++ 6.2 framework. It also reflects on the challenges in implementing OBS and the benefits of updating the simulation model to a modern environment.

Chapter 6 – Conclusion and Future Work

The final chapter summarizes the complete research, emphasizing the successful development of a modern OBS simulation framework. It concludes that burst-assembly plays a critical role in OBS efficiency. The chapter outlines possible future improvements, including adding core-node functionalities, contention resolution, routing strategies, QoS features, and scaling the model into a complete end-to-end OBS network.